FINAL PROJECTTop of Form

DIGITAL WATER LEVEL DISPLAY

Group Members Information

| NAME | REGISTRATION NO. |
| --- | --- |
| - Mirza Amjad Baig | 2025-CS-267 |
| - Ameer Mauwia | 2025-CS-250 |
| - Noraiz Khan | 2025-CS-282 |
| - Zaman Abbas | 2025-CS-285 |
| - Shahzaib Ahmed | 2025-CS-252 |

Instructor Name: Ma’am Aroosh Fatima.

TABLE OF CONTENTS

Abstract	3

Introduction	3

Objectives	3

Components List	3

Theory	4

Priority Encoder (74HC147)	4

Hex Inverter (74HC04)	5

BCD-to-7-Segment Decoder (CD4511)	5

Power Supply (LM7805)	5

Circuit Design	5

Signal Flow	5

Encoder Truth Table	6

Proteus Simulation	6

Proteus Schematic Screenshot	7

Simulation Running	7

Simulation Results Table	8

Hardware Implementation	8

Breadboard Circuit (Full View)	9

IC Close-up (74HC147, 74HC04, CD4511)	9

Display Output (Hardware Test)	10

Working Principle	10

Advantages & Limitations	10

Conclusion	11

# Abstract

This project presents a Digital Water Level Display built using three combinational logic ICs the 74HC147 Priority Encoder, the 74HC04 Hex Inverter, and the CD4511 BCD-to-7-Segment Decoder  assembled on a solderless breadboard. Nine conductive sensor probes placed at different heights inside a water tank detect the water level. The encoder converts the highest active sensor into a 4-bit BCD code; the inverter corrects active-low logic; and the decoder drives a common-cathode 7-segment display to show a digit from 0 (empty) to 9 (full). Power is supplied by a 9 V battery regulated to 5 V by an LM7805. The circuit was designed, simulated in Proteus 8 Professional, and implemented on hardware.

# Introduction

Water is one of the most critical resources in both domestic and industrial environments. Knowing the exact level of water inside a storage tank is essential to avoid overflow, dry-running of pumps, and unnecessary wastage. Traditionally, mechanical float gauges have been used for this purpose; however, they are prone to wear, corrosion, and inaccuracy.

The Digital Water Level Display system replaces mechanical gauges with an all-electronic, digital circuit. It provides an accurate, real-time, single-digit readout on a 7-segment LED display, indicating the water level on a scale of 0 (empty) to 9 (full). This project is particularly significant from the perspective of Digital Logic Design because it integrates three key combinational building blocks.

The project was designed and simulated using Proteus Professional schematic capture and simulation environment. The simulation confirms correct operation for all water levels from 0 to 9.

# Objectives

- Understand and apply the 74HC147 priority encoder in a practical circuit.

- Understand the need for logic-level inversion between encoder and decoder.

- Implement the CD4511 BCD decoder to drive a 7-segment display.

- Assemble the complete circuit on a solderless breadboard.

- Simulate and verify correct operation for all water levels (0–9) in Proteus.

# Components List

| # | Component | Specification | Quantity | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Solderless Breadboard | Standard 830-tie | 1 | Circuit assembly platform |
| 2 | Voltage Regulator | LM7805 (5 V) | 1 | Regulates 9 V battery to 5 V DC |
| 3 | Priority Encoder IC | 74HC147 (Decimal to BCD) | 1 | Encodes water level to 4-bit BCD |
| 4 | Hex Inverter IC | 74HC04 | 1 | Inverts active-LOW BCD to active-HIGH |
| 5 | BCD-to-7-Segment Decoder IC | CD4511 | 1 | Decodes BCD and drives 7-segment display |
| 6 | 7-Segment Display | Common Cathode | 1 | Displays water level digit (0–9) |
| 7 | Resistor | 560 kΩ | 9 | Pull-up resistors for encoder inputs |
| 8 | Resistor | 100 Ω | 1 | Current limiting for 7-segment segments |
| 9 | Capacitor | 104 pF (0.1 µF) Ceramic | 2 | Decoupling caps for LM7805 regulator |
| 10 | Male-to-Male Jumper Wires | Standard | × | IC-to-IC connections on breadboard |
| 11 | Male Header Pins | Standard | × | Sensor probe connection points |
| 12 | Hard Jumper Wire | Solid-core | × | Power and ground rails on breadboard |
| 13 | Battery Clip | 9 V PP3/6F22 type | 1 | Connects battery to circuit |
| 14 | 9 V Battery | PP3 / 6F22 | 1 | Primary power source |

# Theory

## Priority Encoder (74HC147)

The 74HC147 is a 10-to-4 BCD priority encoder. It has 9 active-LOW data inputs (I1- I9) and produces a 4-bit active-LOW BCD output (D, C, B, A). If several inputs are active at the same time, the highest-numbered input wins (priority). When no input is active, the output is 1111  which inverts to 0000 (decimal 0). The pull-up resistors (560 kΩ) keep all unused inputs HIGH.

## Hex Inverter (74HC04)

The 74HC04 contains six independent NOT gates. Since the 74HC147 outputs active-LOW BCD and the CD4511 requires active-HIGH BCD, each of the four BCD bits is passed through an inverter gate to produce the correct logic levels. For example, encoder output 1010 (active-LOW for level 5) becomes 0101 after inversion the correct BCD for decimal 5.

## BCD-to-7-Segment Decoder (CD4511)

The CD4511 accepts a 4-bit BCD input and drives seven segment outputs (a–g), all active-HIGH, for a common-cathode display. Three control pins must be configured for normal operation:

- LT  (Lamp Test, Pin 3) - tie to VCC (HIGH)

- BL  (Blanking,  Pin 4) - tie to VCC (HIGH)

- LE  (Latch Enable, Pin 5) - tie to GND (LOW)

## Power Supply (LM7805)

The LM7805 three-terminal regulator converts the 9 V battery to a stable +5 V DC rail. Two 104 pF (0.1 µF) ceramic capacitors one at the input pin, one at the output pin suppress noise and ensure stability. All IC VCC pins and the display are powered from this +5 V rail.

# Circuit Design

## Signal Flow

| Stage | Component | Function |
| --- | --- | --- |
| Sensing | 9 Probes + 560 kΩ Pull-ups | Input High normally; water contact pulls LOW |
| Encoding | 74HC147 | Active-Low BCD output of highest active sensor |
| Inversion | 74HC04 | Converts active-Low to active-High BCD |
| Decoding & Display | CD4511+7-Seg Display | Drives segments to show level digit 0–9 |

## Encoder Truth Table

| Level | Active Input | Encoder Out (Active-LOW) | After Inversion (BCD) | Display |
| --- | --- | --- | --- | --- |
| 0 (Empty) | None | 1111 | 0000 | 0 |
| 1 | I1 | 1110 | 0001 | 1 |
| 2 | I2 | 1101 | 0010 | 2 |
| 3 | I3 | 1100 | 0011 | 3 |
| 4 | I4 | 1011 | 0100 | 4 |
| 5 | I5 | 1010 | 0101 | 5 |
| 6 | I6 | 1001 | 0110 | 6 |
| 7 | I7 | 1000 | 0111 | 7 |
| 8 | I8 | 0111 | 1000 | 8 |
| 9 | I9 | 0110 | 1001 | 9 |

# Proteus Simulation

The schematic was drawn in Proteus 8 ISIS using the 74HC147, 74HC04, 4511, and 7SEG-COM-CATHO components. LOGICSTATE inputs simulated the sensor probes. The simulation was run for all ten water levels and all displayed correctly.

## Proteus Schematic Screenshot

Figure 1 - Schematic of Digital Water Level Display

## Simulation Running

Figure 2 - Water Level - Display shows '4'

## Simulation Results Table

| Water Level | Inputs LOW | Encoder Out | BCD (After Inversion) | Display | Result |
| --- | --- | --- | --- | --- | --- |
| 0 | None | 1111 | 0000 | 0 | ✔ Correct |
| 1 | I1 | 1110 | 0001 | 1 | ✔ Correct |
| 2 | I2 | 1101 | 0010 | 2 | ✔ Correct |
| 3 | I3 | 1100 | 0011 | 3 | ✔ Correct |
| 4 | I4 | 1011 | 0100 | 4 | ✔ Correct |
| 5 | I5 | 1010 | 0101 | 5 | ✔ Correct |
| 6 | I6 | 1001 | 0110 | 6 | ✔ Correct |
| 7 | I7 | 1000 | 0111 | 7 | ✔ Correct |
| 8 | I8 | 0111 | 1000 | 8 | ✔ Correct |
| 9 | I9 | 0110 | 1001 | 9 | ✔ Correct |

# Hardware Implementation

After successful simulation, the circuit was assembled on a solderless breadboard using the components listed in Section 3. Male header pins were used as water-level sensor probes, and hard jumper wires were used for the power and ground rails. The 9 V battery was connected via a battery clip to the LM7805 regulator input.

## Breadboard Circuit (Full View)

Figure 3 — Assembled breadboard circuit (full view)

## IC Close-up (74HC147, 74HC04, CD4511)

Figure 4 - Close-up of 74HC147, 74HC04, and CD4511 ICs

## Display Output (Hardware Test)

Figure 5 — 7-segment display output during hardware testing

# Working Principle

- Water reaches probe I6 - probes I1 to I6 are all submerged and pulled LOW.

- 74HC147 detects I6 as the highest active input  outputs 1001 (active-LOW BCD for 6).

- 74HC04 inverts 1001  0110 (correct active-HIGH BCD for decimal 6).

- CD4511 decodes 0110  drives segments a, b, d, e, g HIGH  display shows '6'.

# Advantages & Limitations

| Advantages | Limitations |
| --- | --- |
| No microcontroller needed (pure hardware) | Only 9 discrete levels |
| Instantaneous real-time response | Single-digit display only |
| Low cost and easy to assemble | Probes may corrode in water over time |
| Runs on a 9 V battery (portable) | No alarm or pump control output |

# Conclusion

The Digital Water Level Display project successfully demonstrates the combined use of a priority encoder, hex inverter, and BCD-to-7-segment decoder to build a practical, fully digital level-sensing system. All ten levels (0–9) were verified correct in Proteus 8 simulation, and the circuit was successfully implemented on a solderless breadboard. This project reinforces core Digital Logic Design concepts encoder and decoder operation, active-LOW logic, and IC interfacing in a real-world engineering context.